Single-phase Fluid Finite-difference Simulator using Python

B. Manullang, Institut Teknologi Bandung

Current progress in the development is hosted at https://github.com/benjdewantara/fdressim

# Abstract

The author presents the development of a basic finite-difference reservoir simulator using Python as the programming language. The simulator is similar to that of an old traditional simulator, that is to say that it is used to solve a single-phase fluid flow, in a homogeneous and isotropic medium, and discretized in a cartesian coordinate system using a finite-difference approach. The author then gives a few examples with different flow direction (1D, 2D, or 3D).

# Introduction

Often, many undergraduate students majoring in petroleum engineering fail to understand how a reservoir simulator works behind the monitor. Most of these undergraduate students, if not all, must have taken introductory classes on programming, numerical method, and partial differential equations, but still fail to apply them to the field of reservoir simulation. As a result, many of them accept the results at face value without knowing what has happened inside the box. This paper can hopefully give clear explanations on how to utilize those basic knowledges and put them in the form of actual computer program.

Since computers are not able to evaluate the continuous form of a differential equation, we need to approximate the solution to a differential mathematical expression into its discrete form. One method that was widely used is known as finite-difference method. Although most commercial reservoir simulators available nowadays no longer use the traditional finite-difference approach, it is still an eye-opening experience to understand how the governing equation is translated into its finite-difference form. In fact, finite-difference is arguably more intuitive than other discretizing approaches (i.e. corner-point, control-volume).

After we derive the finite-difference form of the differential equation of interest, we then proceed with the presentation of how to put it in the form of computer program. Python is chosen as the programming language because it is easy to understand with its clear syntax.

# Diffusivity Equation for Fluid Flow in Porous Media

In the field of reservoir engineering, the diffusivity equation governs the fluid flow and is derived using Darcy’s law on the basis of conservation of mass. Firstly, one needs to choose the coordinate system that will represent the space. The choice of coordinate system is mainly influenced by the predicted nature of the flow. Due to the radial nature of the fluid flow, the diffusivity equation is usually derived using the cylindrical coordinate system with flow in θ and z direction neglected (see Eq. 1). However, in this paper we will only consider the cartesian coordinate system (see Eq. 2).

| ϕρcT∂P∂t=-1r∂∂rrρkrμ∂P∂r | (1) |
| --- | --- |
| ϕρcT∂P∂t=-∂∂xρkxμ∂P∂x-∂∂yρkyμ∂P∂y-∂∂zρkzμ∂P∂z-ρg | (2) |

# Finite-difference Calculus

The concept of finite-difference forms the foundation of solving differential equations numerically. Instead of solving a differential equation that is supposed to be continuous everywhere in its domain, we look at discrete points and form difference equations. Another way to grasp this concept is that we are not evaluating the behavior of Δfx as Δx gets closer to zero (see Eq. 3) as opposed to the actual definition of derivative (see Eq. 4). This method will introduce some error that depends on the chosen value of h.

| dfdx≈fx+Δx-fxh | (3) |
| --- | --- |
| dfdx=limΔx→0fx+Δx-fxx+Δx-x=limΔx→0ΔfxΔx | (4) |

App. A presents the derivation of diffusivity equation in cartesian coordinate system.

# Python (programming language) and SciPy stack

Python is a programming language designed by Guido van Rossum in 1991. Python is widely known for its neat design, code readability, and its syntax which allows programmers to express concepts in fewer lines of code. It supports object-oriented programming paradigm which enables programmers to express their thinking as objects that interact with each other. Each object may have their own methods to interact with other objects or to perform specific operations.

This simulator makes use of the SciPy stack. SciPy stack is a Python-based ecosystem of open-source software for mathematics, science, and engineering. Two of the core packages which are used in this simulator are NumPy and SciPy library. NumPy provides numerical array objects, and routines to manipulate them. It has also become fundamental package for scientific computing with Python. SciPy library provides numerical routines and is used hand in hand with NumPy.

# Physical Assumptions

Frequently, one will be faced with limitations when modelling a physical system. For example, one cannot know the value of porosity (ϕ) or permeability (k) at every coordinate in space since it is impossible to provide core samples of the entire reservoir system. The assumptions made in the development of this simulator are summarized below

Physical assumptions on fluid:

- There is no change in compositions that make up the fluid. This is also known as a black-oil fluid model.

- Fluid has small, constant compressibility. Highly compressible fluid such as gas is not applicable.

Physical assumptions on rock:

- The behavior of porosity stays the same throughout the reservoir.

Other assumptions:

- Temperature throughout the reservoir stays constant. Therefore, the only variable that affects mass conservation is pressure.

# Designing the Data Structures

The first question we should address is how do we build the computer model? Or specifically, how do we represent the behavior of a reservoir system (consisting of fluid and rock) in a 3D cartesian space using a computer program? Using this reasoning, the author declare classes as sketched in Fig. 1. These classes form the core data structures that model the reservoir. We store the code for these classes in core.py file.

Node

Grid

Fluid

Rock

Reservoir

BoundaryCondition

contains

contains

contains

contains

(optional) assigned to

Figure 1 - Schematic of classes

## Node and Grid Classes

Node objects will be used to represent a point in space. Node will contain information such as the flattened index, the coordinate index (k, j, i notation), whether or not a node is a boundary node, and whether or not a node is a source/sink. Flattened index, in contrast to 3D coordinate index, is a 1D array index. For instance, a Grid object with a dimension of (1, 2, 5) will have one gridblock with respect to z direction, two gridblocks with respect to y direction, and five gridblocks with respect to x direction. Each gridblock is bound to a Node object. Fig. 2 explains how flattened index and coordinate index are assigned to each gridblock for a (1, 2, 5) Grid. This indexing scheme will later be useful when setting up a 2D matrix that consists of linear equations describing pressure relationship among all points at a time level.

(0, 0, 4)

(0, 0, 3)

(0, 0, 2)

(0, 0, 1)

(0, 0, 0)

(0, 1, 4)

(0, 1, 3)

(0, 1, 2)

(0, 1, 1)

(0, 1, 0)

4

3

2

1

0

9

8

7

6

5

x

y

z

Figure 2 - Flattened index and 3D coordinate index in a 1×2×5 Grid

## Fluid and Rock Classes

Fluid in the diffusivity equation presents itself as variables density (ρ) and viscosity (μ). Whereas the porous medium (rock) presents as variables porosity (ϕ) and absolute permeability (k). One should notice that density, viscosity, and porosity are functions of time. Density and porosity can further be combined to form total compressibility (cT), which is equal to compressibility of fluid and compressibility of rock, cT=cl+cr.

| cl=1ρdρdPcr=1ϕdϕdP | (5) |
| --- | --- |

As given by Eq. 5, for any given pressure value, one can determine the value of density and porosity using the following expression:

ρ=ρ0eclP-P0ϕ=ϕ0ecrP-P0

We need to include a reference density or reference porosity with their reference pressure value. For slightly compressible fluid model, this equation of state (EOS) is sufficient. However, for highly compressible fluid, such as gas, the model must take into account other factors such as z-factor. Also, for this simulator, we will consider a constant value of viscosity (μ) at any given pressure value.

## Reservoir class

All the information on Grid, Fluid, and Rock objects will then be passed into a Reservoir object. In addition, when instantiating a Reservoir object, one must also specify the actual dimension of the reservoir (in feet).

## BoundaryCondition class

There are two types of boundary condition used in this simulator, Neumann condition (pressure gradient specified) and Dirichlet condition (pressure specified). App. B explains how each condition is implemented on a boundary node. By default, this simulator will specify a no-flow Neumann condition (∂P∂s=0) at every boundary node when a Grid object is instantiated.

# Applying the Finite-difference Diffusivity Equation

The finite-difference form of diffusivity equation as derived in App. A is put into Python code in file differentiator.py. For each time step, function oneStepDifferentiator will go through each coordinate (Node object) in Grid object and perform what we would like to call as differentiation. The reader may find the code analogous to the derivation in App. A.

Function oneStepDifferentiator will then form as many linear difference equations as the number of Node objects. These linear equations will then be solved implicitly using scipy.linalg function to get pressure distribution for the next time step. Pressure values for each flattened coordinate (as described in Fig. 2) will be generated into a result file.

# 3D Data Visualization

MRST’s plotCellData function is used to visualize pressure distribution at a time level.

# Presentation of Data and Results

# Simulation Examples

We show how to solve for pressure distribution by presenting four example problems. Each problem shows different flow direction. Using the same physical properties, one can observe how the choice of number of grids affect the calculation of pressure distribution. Example 1 specifically address the same problem as given in Ertekin et al. (2001) (Example 5. 11).

## Example 1: 1D-flow (1×1×5 Grid dimension)

Suppose we have a porous medium that can be approximated by a cube shape with 75 ft×1000 ft×5000 ft spatial dimension (see Fig. 3). There exists a constant-rate sink term at x=3,500 ft, y=500 ft. The rock and fluid properties for this problem are: ρl=62lbmft3, cl=3.5×10-6 psi-1, kx=15 mD, ϕ=0.18, and μl=10 cP. With an initial pressure of 6,000 psi and Δt=15 days, determine the pressure distribution during the first year of production.

qsc=-150STBday

1000 ft

75 ft

5000 ft

Figure 3 - Sketch of reservoir

• 1

• 2

• 3

• 4

• 0

∂P∂x=0

∂P∂x=0

75 ft

1000 ft

1000 ft

qsc=-150STBday

Figure 4 - Porous medium and grid block system for Example 1

For this problem, we position the sink term at grid coordinate (0, 0, 3). The resulting pressure distribution by the end of the year is visualized by Fig. 5. It can be seen that the results computed by this simulator agree with Ertekin et al. (2001).

qsc=-150STBday

Figure 5 - Pressure distribution by the end of the year for Example 1

## Example 2: 2D-flow (1×10×20 Grid dimension)

For this problem, we use the same specifications as Example 1. We approximate the location of the sink term at grid coordinate (0, 4, 13). The 3D visualization of pressure distribution is given by Fig. 6.

qsc=-150STBday

Figure 6 - Pressure distribution by the end of the year for Example 2

## Example 3: 3D-flow (5×40×50 Grid dimension)

For this problem, we use the same specifications as Example 1. We approximate the location of the sink term at grid coordinate (0, 19, 34). The 3D visualization of pressure distribution is given by Fig. 7.

qsc=-150STBday

Figure 7 - Pressure distribution by the end of the year for Example 3

## Example 4: 3D-flow (5×51×51 Grid dimension)

In this example, we consider a cube-shaped physical reservoir with 75 ft×5000 ft×5000 ft spatial dimension. And we set the location of the sink term at grid coordinate (0, 24, 24). The sink term has the same constant-rate value as before, qsc=-150STBday. Other specifications remain the same. The 3D visualization of pressure distribution is given by Fig. 8.

qsc=-150STBday

Figure 8 - Pressure distribution by the end of the year for Example 4

# Conclusions

# Acknowledgments

# Nomenclature

# References

Ertekin, T., Abou-Kassem, J. H., and King, G. R. 2001. Basic Applied Reservoir Simulation. Richardson, Texas: Society of Petroleum Engineers, Inc.

Lie, K.-A. 2014. An Introduction to Reservoir Simulation Using MATLAB. Oslo, Norway: SINTEF ICT, Department of Applied Mathematics.

Single-phase Fluid Finite-difference Simulator using Python. https://github.com/benjdewantara/fdressim

# Appendix A - Finite-difference Derivation of Diffusivity Equation

We begin by observing the conservation of mass equation,

| dmcvdt=min-mout+msc | (A1) |
| --- | --- |

Evaluating the right-hand side, for simplicity, we consider mass rate (mx) that only flows in x direction, with term m being ρuA,

min=ρux-Δx2Ax-Δx2mout=ρux+Δx2Ax+Δx2min-mout+msc=ρux-Δx2Ax-Δx2-ρux+Δx2Ax+Δx2+msc

Using the derivative definition as follows,

∂ρuxAx∂x=limΔx→0ρuxAxx-Δx2-ρuxAxx+Δx2x-Δx2-x+Δx2limΔx→0-Δx∂ρuxAx∂x=limΔx→0ρuxAxx-Δx2-ρuxAxx+Δx2

Thus, with a slight abuse of notation, the right-hand side of the equation becomes,

min-mout+msc=-Δx∂ρuxAx∂x+msc

Evaluating the left-hand side of the equation, noticing that mcv=ϕρVb, with term Vb being ΔxΔyΔz,

dmcvdt=dϕρVbdt

Coming back to the earlier mass conservation equation,

dmcvdt=min-mout+mscdϕρVbdt=-Δx∂ρuxAx∂x+msc

Since Vb and Ax are constants,

Vbdϕρdt=-AxΔx∂ρux∂x+mscdϕρdt=-AxΔxVb∂ρux∂x+mscVbdϕρdP∂P∂t=-AxΔxVb∂ρux∂x+mscVbϕρcT∂P∂t=-AxΔxVb∂ρux∂x+mscVb

We now have the general continuity equation. One can generalize the equation further by taking into account the flow in y and z direction as follows,

| ϕρcT∂P∂t=-AxΔxVb∂ρux∂x+-AyΔyVb∂ρuy∂y+-AzΔzVb∂ρuz∂z+mscVb | (A2) |
| --- | --- |

The moment we evaluate the velocity term (us), using Darcy’s law, we will arrive at the diffusivity equation. Again, for clarity we shall derive the equation further by considering only the flow in x direction. Recall the equation for Darcy’s law in some s direction,

us=-kμ∂P∂s-ρg∂z∂s

For flow only in x direction, the continuity equation becomes,

ϕρcT∂P∂t=-AxΔxVb∂∂xρ-kμ∂P∂x+mscVb

We begin translating any ∂∂x term (spatial derivative) using finite-difference method,

ϕρcT∂P∂t=AxΔxVb∂∂xρkμ∂P∂x+mscVb=AxΔxVb1Δxρkμ∂P∂xx+Δx2-ρkμ∂P∂xx-Δx2+mscVb=AxVbρkμx+Δx2∂P∂xx+Δx2-ρkμx-Δx2∂P∂xx-Δx2+mscVb=ρkAxμVbx+Δx21Δx Px+Δx-Px-ρkAxμVbx-Δx21Δx Px-Px-Δx+mscVbϕρcT∂P∂t=ρkAxμVbΔxx+Δx2Px+Δx-Px-ρkAxμVbΔxx-Δx2Px-Px-Δx+mscVb

We can group the term ρkAxμVbΔxx and define them as transmissibility, Tx,

ϕρcT∂P∂t=Tx+Δx2Px+Δx-Px-Tx-Δx2Px-Px-Δx+mscVb=Tx+Δx2Px+Δx-Tx+Δx2+Tx-Δx2Px+Tx-Δx2Px-Δx+mscVb

We then proceed by translating the ∂∂t term (time derivative) using finite-difference. We also assign superscript t or t+Δt to any variable to specify the time level.

| ϕρcTPxt+Δt-PxtΔt=Tx+Δx2tPx+Δxt+Δt-Tx+Δx2t+Tx-Δx2tPxt+Δt+Tx-Δx2tPx-Δxt+Δt+mscVbϕρcTΔtPxt+Δt-Pxt=Tx+Δx2tPx+Δxt+Δt-Tx+Δx2t+Tx-Δx2tPxt+Δt+Tx-Δx2tPx-Δxt+Δt+mscVb | (A3) |
| --- | --- |

We have arrived at the final finite-difference form of diffusivity equation for flow only in x direction. One should notice that any Px term encountered so far is actually P at some x, y, z coordinate (i.e. Px,y,z). Similarly, the term Px+Δx actually denotes Px+Δx,y,z. The subscript y,z is left out for brevity when considering the differential with respect to x. We can further generalize this form and factor in the flow terms for y and z direction as follows,

| differentialin t=differentialin x direction+differentialin y direction+differentialin z direction+mscVb | (A4) |
| --- | --- |

where:

differentialin x direction=Tx+Δx2tPx+Δxt+Δt-Tx+Δx2t+Tx-Δx2tPxt+Δt+Tx-Δx2tPx-Δxt+Δtdifferentialin y direction=Ty+Δy2tPy+Δyt+Δt-Ty+Δy2t+Ty-Δy2tPyt+Δt+Ty-Δy2tPy-Δyt+Δtdifferentialin z direction=Tz+Δz2tPz+Δzt+Δt-Tz+Δz2t+Tz-Δz2tPzt+Δt+Tz-Δz2tPz-Δzt+Δt+-Tz+Δz2+Tz-Δz2ρgΔzdifferentialin t=ϕρcTΔtPx,y,zt+Δt-Px,y,ztTs=ρkAsμVbΔss

# Appendix B - Implementing Boundary Condition

The finite-difference derivation as described in App. A is based on the central difference scheme. This type of scheme is only applicable to a gridblock which has neighboring gridblocks in its x, y, and z direction. However, this is not the case for a boundary gridblock. This situation is best described using a 3D grid sketch (Fig. B1). For a boundary gridblock, a special treatment based on either Dirichlet or Neumann boundary condition needs to be made.

Figure B1 - Sketch of a boundary node (left) and a regular node (right) with their neighboring gridblocks

We restrict further discussion on the flow equation only looking at flow in x direction for simplicity. For instance, consider a boundary gridblock at x that does not have both gridblocks at x-Δx2 and x+Δx2 to interact with. This means with respect to x, either Px+Δx or Px-Δx is missing and the diffusivity equation (Eq. A4) cannot be completed. Either one of them is the boundary pressure. We need to evaluate how Dirichlet or Neumann condition is imposed on this boundary pressure.

- Dirichlet condition (pressure specified)

Suppose the boundary pressure is Px-Δx, for Dirichlet condition, one can directly specify the boundary pressure,

Px-Δx=C

The value of C is some constant but can also be a function of time. But we will assume it is constant throughout the time.

- Neumann condition (pressure gradient specified)

For this condition, we are given the gradient value, ∂P∂x, which is equal to C. We can then translate this gradient into the following,

∂P∂x=C∂P∂x=Px-Px-ΔxΔxPx-Δx=Px-CΔx

Similarly, if the boundary pressure is Px+Δx, the gradient expression can be approximated as follows,

∂P∂x=Px+Δx-PxΔxPx+Δx=Px+CΔx

For a no-flow condition, the gradient is zero (C=0), thus the boundary pressure is directly equal to Px.